FORM 2

PROVISIONAL SPECIFICATION

[See Section 9, Rule 13 — Patents Act 1970 & Patents Rules 2003]

THE PATENT OFFICE, INDIA

─────────────────────────────────────────────────────────────────

TITLE OF THE INVENTION:

ORBITGNN: A PHYSICS-INFORMED GRAPH NEURAL NETWORK SYSTEM AND METHOD
FOR SATELLITE MANOEUVRE DETECTION AND DELTA-VELOCITY ESTIMATION
USING PUBLICLY AVAILABLE TWO-LINE ELEMENT ORBITAL DATA

─────────────────────────────────────────────────────────────────

| Inventors | Chunduri Sai Kaushik Keshav Gujrathi |
| --- | --- |
| Applicant | [INSTITUTION NAME] |
| Application No. | To be allotted by Patent Office |
| Date of Filing | ________________ |
| Specification Type | Provisional Specification |
| Technical Field | Space Technology / Machine Learning / Satellite Orbit Determination |

1. FIELD OF THE INVENTION

The present invention relates to the technical fields of space situational awareness, satellite orbit determination, and machine learning. More particularly, the invention provides a system and method — designated herein as "OrbitGNN" — that integrates orbital physics propagation with Graph Neural Networks (GNNs) to automatically detect satellite orbital manoeuvres and estimate the associated delta-velocity (ΔV) using only publicly available Two-Line Element (TLE) data, without requiring access to proprietary satellite telemetry, ground-station measurement data, or onboard propulsion system information.

The invention has direct applications in space traffic management, collision avoidance, debris monitoring, satellite health assessment, manoeuvre characterisation, and autonomous constellation management.

2. BACKGROUND OF THE INVENTION

2.1  The Problem

As of 2024, more than 27,000 artificial objects are catalogued in Earth orbit. Active satellites periodically perform orbital manoeuvres — controlled propulsion burns — for station-keeping, collision avoidance, or ground-track adjustments. When a manoeuvre occurs, ground operators must be alerted promptly to update conjunction screening data. A delay of even a few hours increases collision risk and can contribute to the Kessler cascade. The primary source of publicly available orbital data — the Two-Line Element (TLE) set — provides six mean Keplerian elements (a, e, i, Ω, ω, M) for each satellite at irregular publication intervals, typically 6–36 hours after a manoeuvre occurs.

2.2  Limitations of Existing Methods

Prior art methods for TLE-based manoeuvre detection have five critical limitations:

(a) Synthetic data only: All known ML approaches train and test on synthetically generated TLEs with idealised noise models that do not reflect real TLE publication cadences or actual radar/optical measurement noise.

(b) No orbital physics: Raw-TLE ML methods treat Keplerian elements as plain numerical features, ignoring the governing physics equations, leading to high false alarm rates from normal secular orbital drift.

(c) No peer satellite context: No prior method exploits the diagnostic signal that peers in the same orbital regime experience identical environmental perturbations. An anomaly in one satellite but not its peers is a strong manoeuvre indicator.

(d) Fixed propagation Δt: All prior art assumes a fixed 24-hour propagation step, introducing systematic errors proportional to the deviation between actual and assumed inter-TLE elapsed time — especially critical for mean anomaly propagation.

(e) No ΔV estimation: No prior TLE-based method derives the manoeuvre magnitude in m/s. This has previously required proprietary propulsion telemetry or high-fidelity orbit determination software.

3. OBJECTS OF THE INVENTION

The principal objects of the present invention are:

1.  To provide a system and method for detecting satellite orbital manoeuvres using only publicly available TLE data, without any proprietary telemetry or ground-station data.

2.  To provide a physics residual computation method using the actual inter-TLE elapsed time Δt for J₂-corrected Keplerian propagation, eliminating systematic errors from fixed-Δt assumptions.

3.  To provide a Transformer-based ResidualEncoder that encodes temporal sequences of orbital physics residuals, distinguishing normal orbital evolution from manoeuvre signatures.

4.  To provide an orbital-shell-aware graph construction method connecting satellites within the same orbital regime (GEO/SSO/LEO) with no cross-shell edges, enabling physically meaningful peer-context aggregation.

5.  To provide a composite anomaly scoring method combining self-forecast error, peer-graph-forecast error, and MC-Dropout uncertainty, z-normalised across the satellite batch at each time step.

6.  To provide a ΔV estimation method from TLE-derived semi-major axis residuals via the linearised Gauss tangential burn equation, without propulsion telemetry.

7.  To provide a self-supervised training procedure requiring no labelled manoeuvre annotations, applicable to any satellite with public TLE data.

4. STATEMENT OF THE INVENTION

According to the present invention, there is provided a computer-implemented system and method for satellite manoeuvre detection and delta-velocity estimation, comprising the following operatively connected modules:

Module 1 — TLE Loading & Grid Snapping: Receives TLE records for S satellites; extracts six Keplerian elements; maps to a regular daily grid; records actual elapsed time Δt between consecutive TLE epochs; and generates a binary validity mask.

Module 2 — Physics Residual Computation: Propagates orbital elements using J₂-corrected Keplerian model with actual Δt; computes signed residuals δ = a_obs(t+1) − â(t+1); applies angular wrapping to (−π, π]; applies per-satellite z-normalisation on training data.

Module 3 — ResidualEncoder (Transformer): A Transformer encoder (d=64, 4 heads, 2 layers, GELU, dropout=0.2) that maps a window of W=8 residuals per satellite to a latent embedding and self-forecast ŷ^self ∈ ℝ⁶.

Module 4 — Orbital-Shell-Aware Graph: Assigns each satellite to GEO/SSO/LEO shell; builds k-NN (k=3) adjacency within each shell using mean-motion similarity; creates NO cross-shell edges.

Module 5 — NeighborGCN: Degree-normalised graph convolution H' = GELU(D⁻¹AHW) + H aggregates peer embeddings; outputs peer-context forecast ŷ^peer ∈ ℝ⁶.

Module 6 — Composite Anomaly Scoring: Executes 15 MC-Dropout passes for uncertainty σ̄; computes score = w₁·z(‖y−ŷ^self‖²) + w₂·z(‖y−ŷ^peer‖²) + w₃·z(‖σ̄‖²) with z-normalisation across satellite batch.

Module 7 — ΔV Estimation: Extracts Δa at alarm step; computes n = √(μ/a³); estimates ΔV ≈ (n·|Δa|)/2 [m/s] via linearised Gauss equation without propulsion telemetry.

5. DETAILED DESCRIPTION OF THE PREFERRED EMBODIMENT

5.1  TLE Loading and Grid Snapping

TLE archive files (Space-Track.org format) are parsed for each satellite. Six mean Keplerian elements are extracted at each epoch. Each TLE is mapped to the nearest daily grid point within ±48 hours. The actual elapsed time Δt_{k,t} in seconds between consecutive TLE epochs is computed and stored. Grid steps without a TLE within ±48 h are marked invalid (v_{k,t} = 0).

Key Innovation: The actual inter-TLE Δt is used for propagation, not a fixed 24-hour step. For a LEO satellite with a 90-minute period, a 6-hour error in Δt produces ~4 full orbital periods of error in the mean anomaly — completely overwhelming the manoeuvre signal. Correct Δt computation is essential for reliable residual computation.

5.2  J₂-Corrected Keplerian Physics Residuals

The J₂ secular perturbation rates are:

n = √(μ/a³)                              [mean motion, rad/s]

p = a(1 − e²)                            [semi-latus rectum, km]

dΩ/dt = −(3/2)·n·J₂·(Rₑ/p)²·cos(i)    [RAAN drift, rad/s]

dω/dt =  (3/4)·n·J₂·(Rₑ/p)²·(5cos²i−1) [AoP drift, rad/s]

dM/dt =  n                                [mean motion propagation]

Constants: J₂ = 1.08262668×10⁻³, Rₑ = 6378.137 km, μ = 398600.4418 km³/s².

The signed residual:

δ_{k,t}  =  a_obs_{k,t+1}  −  â_{k,t+1}

Angular wrapping: wrap(θ) = ((θ + π) mod 2π) − π ensures residuals ∈ (−π, π].

5.3  ResidualEncoder (Transformer)

Input: window X_k ∈ ℝ^{W×6}, W=8. Architecture: Linear(6→64) projection → Learned positional embedding → 2× Transformer encoder layers (d=64, 4 heads, FF=128, GELU, dropout=0.2) → Linear forecast head (64→6). Output: summary h_k ∈ ℝ^64, self-forecast ŷ^self_k ∈ ℝ^6. Total trainable parameters: 105,100.

5.4  Orbital-Shell-Aware Graph (Key Innovation)

| Shell | Name | Altitude | Inclination |
| --- | --- | --- | --- |
| 0 | GEO | >30,000 km | Any |
| 1 | SSO / Polar | 600–1,000 km | >88° |
| 2 | LEO-66° | 1,000–1,700 km | 60°–70° |

k-NN (k=3) edges connect satellites within the same shell only. No cross-shell edges. This ensures GEO manoeuvre signatures are not propagated to SSO/LEO peers, which experience entirely different perturbation environments (different atmospheric drag, different solar radiation pressure, different lunisolar gravity gradients).

5.5  NeighborGCN

H'  =  GELU( D⁻¹ · A · H · W )  +  H

D = diagonal degree matrix; W ∈ ℝ^{64×64} learned; residual connection preserves self-information. A peer forecast head (Linear 64→6) maps H' to ŷ^peer ∈ ℝ^6.

5.6  Composite Anomaly Scoring (Key Innovation)

score_k  =  w₁·z(‖y_k−ŷ^self‖²)  +  w₂·z(‖y_k−ŷ^peer‖²)  +  w₃·z(‖σ̄_k‖²)

z(x_k) = (x_k − mean{x}) / (std{x} + ε) across satellite dimension at each step. σ̄_k = element-wise std of 15 MC-Dropout peer forecast samples (epistemic uncertainty). Default: w₁=w₂=w₃=1.0. Z-normalisation makes the score relative within the constellation, automatically accounting for time-varying global perturbations.

5.7  ΔV Estimation (Key Innovation)

ΔV  ≈  ( n · |Δa| ) / 2          [m/s]

Derived from linearised Gauss variational equation for near-circular tangential burns. n = √(μ/a³) [rad/s], |Δa| [km] from TLE-derived residual. Accuracy: ±50% for tangential burns. No propulsion telemetry required. First known method to provide ΔV estimates from public TLE data alone.

5.8  Training Procedure

Loss  =  MSE(ŷ^self, y_next)  +  MSE(ŷ^peer, y_next)

Self-supervised — no manoeuvre labels used during training. Chronological split: Train (60%): 2020-01-10→2021-03-17; Validation (20%): 2021-03-18→2021-08-09; Test (20%): 2021-08-10→2022-01-01. No temporal data leakage.

6. BEST MODE OF CARRYING OUT THE INVENTION

6.1  Dataset

Validated on the TLE Observation Benchmark Dataset (Shorten & Abdurahimov, 2023) — the first public real TLE benchmark with verified manoeuvre timestamps.

| Satellite | Shell | Altitude (km) | Inclination | Manoeuvres |
| --- | --- | --- | --- | --- |
| Fengyun-2F | GEO | 35,786 | 4.4° | 12 |
| Fengyun-2H | GEO | 35,786 | 4.4° | 5 |
| Fengyun-4A | GEO | 35,786 | 0.2° | 26 |
| CryoSat-2 | SSO/Polar | 717 | 92.0° | 14 |
| SARAL | SSO/Polar | 781 | 98.5° | 4 |
| Sentinel-3A | SSO/Polar | 814 | 98.6° | 25 |
| Sentinel-3B | SSO/Polar | 814 | 98.6° | 10 |
| Jason-3 | LEO-66° | 1,336 | 66.0° | 4 |
| Sentinel-6A | LEO-66° | 1,336 | 66.0° | 7 |
| TOTAL | — | — | — | 125 (107 train + 30 test) |

6.2  Multi-Seed Performance (5 Seeds, Test Split)

| Seed | ROC-AUC | PR-AUC | F1 | Precision | Recall |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.5880 | 0.0757 | 0.1840 | 0.1429 | 0.2586 |
| 2 | 0.5720 | 0.0861 | 0.2044 | 0.1772 | 0.2414 |
| 3 | 0.5877 | 0.0747 | 0.1867 | 0.1522 | 0.2414 |
| 4 | 0.5815 | 0.0807 | 0.1752 | 0.1519 | 0.2069 |
| 5 | 0.6036 | 0.0907 | 0.1867 | 0.1522 | 0.2414 |
| Mean±Std | 0.587±0.010 | 0.082±0.006 | 0.187±0.010 | 0.155±0.012 | 0.238±0.017 |

Bootstrap 95% CI (N=2000): ROC-AUC [0.482, 0.660]; PR-AUC [0.045, 0.120]; F1 [0.110, 0.256].

6.3  Comparison with Baselines

| Method | ROC-AUC | PR-AUC | F1 | Causal? |
| --- | --- | --- | --- | --- |
| ResidMag | 0.562 | 0.050 | 0.106 | Yes |
| RollingZ | 0.505 | 0.050 | 0.094 | Yes |
| IsolationForest | 0.575 | 0.122 | 0.201 | No (non-causal batch) |
| OrbitGNN (this invention) | 0.587±0.010 | 0.082±0.006 | 0.187±0.010 | Yes |

Statistical significance: OrbitGNN vs ResidMag: p=0.0084, Cohen's d=2.16 (large effect). OrbitGNN vs RollingZ: p<0.001, Cohen's d=7.03 (very large effect). OrbitGNN significantly outperforms all causal baselines.

6.4  Event-Level Detection Rate (±72 hours)

| Tolerance Window | Detection Rate | False Alarms / Day |
| --- | --- | --- |
| ±24 h | 39.3% ± 3.3% | 0.29 |
| ±48 h | 51.3% ± 7.5% | 0.27 |
| ±72 h | 61.3% ± 8.6% | 0.24 |

6.5  ΔV Estimates for Test-Split Manoeuvres

| Satellite | Manoeuvre Date | ¦Δa¦ (km) | ΔV Estimate (m/s) |
| --- | --- | --- | --- |
| Fengyun-2F | 2021-08-04 | 0.196 | 0.007 |
| Fengyun-2F | 2021-09-22 | 0.266 | 0.010 |
| Fengyun-4A | 2021-08-20 | 0.091 | 0.003 |
| Fengyun-4A | 2021-09-21 | 0.192 | 0.007 |
| Fengyun-4A | 2021-11-12 | 0.267 | 0.010 |
| Sentinel-3A | 2021-09-15 | 0.031 | 0.011 |

These ΔV values are consistent with published literature on GEO station-keeping burns (0.005–0.25 m/s), validating the estimation method.

6.6  Software & Reproducibility

Implementation: Python 3.11, PyTorch 2.x. Repository: https://github.com/keshavgujrathi/OrbitGNN (branch: real-tle-pipeline). 4,552 lines across 5 core modules. 157 automated unit tests — all pass. GitHub Actions CI on Python 3.10/3.11/3.12. Model: 105,100 parameters (CPU-deployable).

7. CLAIMS

We claim:

Claim 1: A computer-implemented method for detecting orbital manoeuvres of Earth-orbiting satellites, the method comprising the steps of: (a) receiving Two-Line Element (TLE) records for a plurality of satellites from a public orbital data source and extracting six mean Keplerian orbital elements therefrom; (b) computing, for each satellite and each consecutive pair of TLE observations, a physics residual by propagating orbital elements forward using the actual elapsed time in seconds between the two TLE epochs via a J₂-corrected Keplerian propagation model, and computing the signed difference between the propagated and observed elements, with angular wrapping applied to angular components; (c) encoding temporal sequences of the said physics residuals using a Transformer neural network to produce a latent embedding and a self-forecast of the next residual step per satellite; (d) constructing an orbital-shell-aware adjacency graph wherein edges connect satellites within the same orbital regime using orbital element similarity, and no edges are created between satellites in different orbital regimes; (e) aggregating peer satellite embeddings via graph convolution over the said graph to produce a peer-context forecast per satellite; (f) computing, for each satellite, a composite anomaly score as the weighted sum of zero-mean unit-variance normalised self-forecast error, peer-forecast error, and Monte Carlo Dropout epistemic uncertainty, wherein the normalisation is performed across the satellite batch at each time step; and (g) classifying satellites whose anomaly score exceeds a threshold as having performed an orbital manoeuvre.

Claim 2: The method as claimed in Claim 1, wherein step (b) uses the actual elapsed time Δt in seconds between consecutive TLE epochs as the propagation time step, computed from the difference in TLE epoch timestamps, in place of any fixed nominal time step, thereby eliminating systematic errors in the propagated mean anomaly residual arising from irregular TLE publication cadences.

Claim 3: The method as claimed in Claim 1, wherein step (d) comprises: assigning each satellite to an orbital shell from a group comprising geostationary orbit (GEO, altitude > 30,000 km), sun-synchronous orbit (SSO, altitude 600–1,000 km and inclination > 88°), and low Earth orbit (LEO-66°, altitude 1,000–1,700 km and inclination 60°–70°); constructing k-nearest-neighbour edges within each shell using mean-motion similarity; and creating no edges between satellites in different shells.

Claim 4: A method for estimating the delta-velocity of a satellite manoeuvre from publicly available Two-Line Element data without propulsion telemetry, the method comprising: detecting a manoeuvre event at time step t for a satellite k using the method of Claim 1; extracting the semi-major axis residual Δa in kilometres from the physics residual at step t; computing mean motion n = √(μ/a³) in radians per second; and estimating the tangential delta-velocity as ΔV ≈ (n·|Δa|)/2 in metres per second using the linearised Gauss variational equation for near-circular orbits.

Claim 5: The method as claimed in Claim 1, wherein the composite anomaly score of step (f) is: score_k = w₁·z(‖y_k − ŷ^self_k‖²) + w₂·z(‖y_k − ŷ^peer_k‖²) + w₃·z(‖σ̄_k‖²), wherein z(·) denotes normalisation (x − mean) / (std + ε) across the satellite batch; ŷ^self is the Transformer encoder self-forecast; ŷ^peer is the graph convolution peer-context forecast; σ̄_k is the standard deviation over Monte Carlo Dropout samples representing epistemic uncertainty; and w₁, w₂, w₃ are positive real coefficients.

8. ABSTRACT

The present invention provides OrbitGNN — a physics-informed Graph Neural Network system and method for automatic satellite manoeuvre detection and delta-velocity (ΔV) estimation using only publicly available Two-Line Element (TLE) orbital data. The invention computes J₂-corrected Keplerian physics residuals using the actual inter-TLE elapsed time, encodes residual sequences via a Transformer architecture (ResidualEncoder), aggregates peer satellite context via an orbital-shell-aware Graph Convolutional Network (NeighborGCN) with no cross-regime edges, and generates per-satellite anomaly scores combining self-forecast error, peer-forecast error, and Monte Carlo Dropout uncertainty, z-normalised across the satellite batch. A novel ΔV estimator derives manoeuvre magnitude from TLE-derived semi-major axis residuals via the linearised Gauss tangential burn equation — requiring no propulsion telemetry. Validated on the real TLE Observation Benchmark Dataset (9 satellites, 2020–2022, 125 verified manoeuvre events), the invention achieves ROC-AUC 0.587 ± 0.010 and detects 61.3% ± 8.6% of manoeuvre events within ±72 hours — statistically significantly superior to residual-magnitude thresholding (p = 0.0084, Cohen's d = 2.16). The system is self-supervised (no labels required), fully causal (real-time usable), and runs on commodity hardware (105,100 parameters).

SIGNATURES OF INVENTORS

We, the inventors named herein, declare that the information provided in this Provisional Specification is true and correct to the best of our knowledge and belief, and that we are the true and first inventors of the invention described herein.

| FIELD | INVENTOR 1 | INVENTOR 2 |
| --- | --- | --- |
| Full Name | Chunduri Sai Kaushik | Keshav Gujrathi |
| Designation | [Roll No. / Programme] | [Roll No. / Programme] |
| Signature | _____________________ | _____________________ |
| Date | ____ / ____ / 2024 | ____ / ____ / 2024 |

Place of Signing: _______________________________

Date of Signing:  ____ / ____ / 2024

─────────────────────────────────────────────────────────────────

Note: This Provisional Specification establishes the priority date. A Complete Specification must be filed within TWELVE (12) MONTHS per Section 9(1) of the Patents Act, 1970. Failure to file within 12 months will result in abandonment.